ARTIFICAL INTELLIGENCE APPROACH FOR FAST PACED POWDER FORMULATION AND DEVELOPMENT

BY

AKINYEMI TEMINIJESU JESUFEMI

160407501

SUBMITTED TO

THE DEPARTMENT OF SYSTEMS ENGINEERING

UNIVERSITY OF LAGOS

IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR

THE AWARD OF BACHELOR OF SCIENCE (B.Sc) DEGREE

IN SYSTEMS ENGINEERING, UNIVERSITY OF LAGOS, AKOKA LAGOS.

OCTOBER, 2021

CERTIFICATION

This is to certify that this project “ARTIFICAL INTELLIGENCE APPROACH FOR FAST PACED POWDER FORMULATION AND DEVELOPMENT” was carried out by AKINYEMI TEMINJESU JESUFEMI with matriculation number 160407501 of the department of Systems Engineering, University of Lagos, under our supervision.

………………………….                                                                   …………………………….

DR. O.A GEORGE                                                                                  	       Date

(PROJECT SUPERVISOR)

………………………….                                                                   …………………………….

DR. T.T AKANO                                                    		                    Date

Head of Department

Systems Engineering

DEDICATION

I dedicate my project work to God who in his abundant mercy and infinite grace has sustained me and helped me this far. I also dedicate it to my amazing parents, Mr and Mrs Akinyemi, my siblings and friends who have been ever loving and supportive in all I do.

ACKNOWLEDGEMENT

This part of my life has brought me tremendous delight since it has enhanced my understanding of research and general intuition about learning and application.

I owe a huge debt of gratitude to my family, who provided me with invaluable inspiration, support, and prayers throughout this period. Without them, this study would not have been able to be completed.

My sincere appreciation goes to my supervisor Dr. O. A. George for the knowledge and skills I acquired throughout this experience.

To all my lecturers in Systems Engineering department, University of Lagos, I say a big thank you for the outstanding knowledge that I have acquired so far.

My heartfelt appreciation goes to my fellow students at the University of Lagos for their unending assistance and contribution to my educational and social growth.

Overall, I thank Almighty God for this wonderful opportunity and for providing me with the life, power, and zeal necessary to complete this study project despite all of the societal difficulties encountered along the way.

ABSTRACT

No doubt, the popularity of artificial intelligence has evolved so significantly in the past one decade. In food technology it has helped to monitor every stage of the production process and make significant predictions. With increasing focus on digitalized foods to suit diverse and fast-paced consumer needs, it has further opened myriad of opportunities in product discovery and development and research. More so, in the analyses of droplet drying to the determination of determination of material properties and microstructural characterization. Despite the availability of abundant samples of drying data, the real-time powder processing and control still rely heavily on traditional approaches; unsuitable for keeping-up with the present-day consumers’ needs and trends. Certainly, the development of a method that can predict correctly the material properties would already be a monumental milestone in the powder processing sector of food processing.  The aim of this study thus, is to assess the existing drying techniques and pilot discussions into the application of machine learning algorithms for drying for the purpose of new product development. This study was able to show that the Long short-term memory (LSTM), an artificial recurrent neural network (RNN) is good enough to predict the drying kinetics for food powder. The results obtained showed that this Artificial neural network methodology could precisely predict the drying kinetics for lactose, whole milk, skim milk food powder with high R2 score   within the range of 0.93-1.

TABLE OF CONTENTS

Pages

Title  page											i

Certification 											ii

Dedication 											iii

Acknowledgement										iv

Abstract											v

Table of Contents										vi

List of Tables											viii

CHAPTER ONE: INTRODUCTION

1.1	Background of the Study								1

1.2	Problem Statement									4

1.3	Proposed solution									5

1.4	Aim											6

1.5	Objectives										6

1.6	Solution approach									6

1.7	Significance of the project								7

CHAPTER TWO: LITERATURE REVIEW

2.1	Supervised learning									8

2.2	Unsupervised learning								9

2.3	Semi supervised learning								10

2.4	Reinforcement learning								10

2.5	Representation learning 								11

2.6	Machined learning models								11

2.6.1	Supervised learning models								11

2.6.2	Classification										12

2.7	Unsupervised learning models							13

2.7.1	Clustering										13

2.8	Artificial neural network								13

2.9	Types of neural network								15

2.9.1	Feed forward Neural network								15

2.9.2	Convolution neural network (convent)						15

2.9.3	Recurrent Neural Networks (RNN)							15

CHAPTER THREE: METHODOLOGY

3.1	Data collection									19

3.2	Data preparation									19

3.3	Data Normalization 									20

3.5	Creation of training and testing set of data						21

3.6	Seeding 										21

3.7	Model											23

3.7.1	Create environment 									25

3.7.2	Define network									26

3.7.3	Compile network									27

3.7.4	Fit network										28

3.7.5	Evaluate network									28

3.7.6	Make Prediction 									29

CHAPTER FOUR: RESULTS AND DISCUSSION

4.1	Introduction 										30

4.2	Effects of keeping neuron and window constant and changing layers		31

4.3	Effects of keeping layers and neurons constant and changing the window size	34

4.4	Effects of keeping layers and window constant and changing neurons		37

4.5	Temperature Vs Time									40

4.6	Mass Vs time										44

CHAPTER FIVE: CONCLUSION								46

Reference											47

LIST OF FIGURE

Pages

Fig. 3.1	Code snippet for the seeding process 						23

Fig. 3.2	LSTM Cell diagram 								24

Fig.3.3		Code snippet for the model definition					27

Fig 3.4		Code snippet for the model compilation 					28

CHAPTER ONE

INTRODUCTION

1.1 BACKGROUND

In recent times especially the corona virus global pandemic food security has been a major issue. Food scientist have thought of ways to preserve food and also, ongoing research is being made for the development of new powdery food products. The drying process although very ancient is one of the most important methods for preserving food. The interest of consumers towards dehydrated product continues to rise as a result of increasing awareness of chemical sounding preservatives. Although drying may look simply, it involves a lot of processes.

Drying is the process by which water or moisture is removed from a solid, semisolid or liquid by the process of evaporation.  Drying process usually requires a source of heat and a sink to receive the vapor produced during the process. It is a process that involves heat and mass transfer.

According to (1) drying as a process usually prevents the growth and production of microorganisms which causes decay and minimizes further meditated deteriorative reactions. It also helps to elongate the shell life of materials, minimizes packaging and improve storage for transportation.(2).

In the food Industry the use of drying process is common as it improves the stability of food and minimizes both physical and chemical reaction changes during storage.(3).

Also, in recent times food consumers have inclined towards dried food product, this is because of the increasing awareness about chemical preservatives and food addictiveness and their effect on the human system

Hence the drying technique although an ancient technique is an important process which is extensively used in food preservation.

The Drying process can be achieved in various ways, but it is important to know that the choice of method depends on the material itself and the sanitary level acquired (4).

METHODS OF DRYING

Natural Air Drying

This process involves the drying of material with unheated, forced air. It takes due advantage of natural drying potential of air. This process must take into consideration the air temperature, relative humidity and moisture content and temperature of material being dried. It is a slow process and usually weather dependent (5). Natural air drying is considered to be the simplest and economical way to process grains, fruits and vegetables.

Heated Air Drying

This is common technique which involves heated stream of air heating up a material by convention and carrying away the vapor by humidity. In this process the water level in a material is reduced in order to inhibits the growth of microorganism and reduce the rate at which both enzymatic and non-enzymatic reactions occur (1). Air temperature must be monitored and controlled, based on the quality of the product because air that is too hot can lead to dehydration of the of the slid surface and crust formation .

Freeze drying

Freeze drying also known as Lyophilization is the process in which water is sublimed from a product after it has been frozen.(6).The solution  is first frozen prior to drying and the solvent which is in sloid state is then converted to gas phase directly (that is sublimed) below the melting point of the solvent .This very process is mostly carried out under reduced pressure in order to allow drying to proceed at a reasonable rate .Freeze drying is  regarded as the best method to preserve the biological activity of protein and vitamins as such it is now  commonly used to preserve food like backpackers travelling to remote areas.

Supercritical drying (superheated steam drying)

Supercritical drying is the steam drying of products containing water. This is possible using steam   because as the water in the product being dried is boiled off, it combines with the drying medium and increases its flow. This method is mostly employed in a closed circuit which allows a proportion of latent heat to be recovered by recompression. Supercritical drying as a lot of potential of drying food if done at a reduced pressure and lower boiling point.

Spray drying

Spray drying enables the transformation of feed from a fluid state into a dried particular form by spraying the feed into the hot drying medium (1). In this method, feed which is to be pumped is first converted to fog of droplets (that is atomized) each about 100 micrometers in diameter and then the droplets dry’s quickly as it falls under gravity which is accompanied by heated air. The product that is the dried particles exit through the bottom of the dryer and are then separated from the drying air by using a system of cyclones, plus bags filter of electrostatic precipitators.

Spray drying as found it most application in the production of milk powder, tomato powder and in the pharmaceutical industry where there are quality standards to be meet, end-product must comply with standardized particle size distribution, residual moisture content, bulk density and morphology (1).

Thin layer Drying

Thin layer Drying is the process of drying one layer of a sample particles or slice. This layer will have a thin structure and because of this the temperature distribution can be assumed to be uniform. This method of drying is very suitable for lumped parameter models.

The most common of these methods are the Heated Air drying or Natural Air drying. However, this process takes longer drying time, have lower energy efficiency and poor-quality attributes.(7). Most of these problems are usually due to an uncontrolled drying process and improper product development strategies.

In the food powder processing industries drying is usually carried out to determine the drying kinetics of droplets. This drying process is often carried out by laboratory experiments for example the spray drying and thin layer drying but this laboratory experiment is often time consuming and relatively expensive.

Physical based and theoretical models have also been impactful in drying but it is very complex and requires significant scientific research and consumes time. Several important and fundamental information such as the structural configuration of a wide and diverse range of food materials and their unique properties are required. Also, their temperature distribution and their relationship with moisture content are necessary to build and develop such physics-based models (8).

In addition, the optimization of process parameters which includes drying time, level of water activity, drying air velocity, sample geometry, drying temperature and relative humidity, are all required for the control of the drying operation for ensuring better quality product (9)

1.2 PROBLEM STATEMENT

In food processing, despite the availability of abundant samples of drying data, the real-time powder processing and control still rely heavily on traditional approaches. According to (Imran, Shyam, Joardder, Karim., 2020), Agri-based dried food product are developed and drying systems are designed based on empirical knowledge or by trial and error without taking into cognizance the underlying micro level fundamental drying mechanism. As a result of this there have to be a fundamental understanding of the drying process.

Drying is a very complicated process and if the current mathematical models can efficiently describe the drying process would be really beneficial to the food processing industries.

According to (10) there are two spectrums of current mathematical model, some aspect of this mathematical models uses empirical modeling but despite the simplicity of the empirical modelling it does not accurately predict the physics of the drying phenomena. It also very important to know that these empirical models can only operate within a small range of validity, this makes it redundant from a design perspective.

The other spectrum of this mathematical model is the one deeply rooted in physics. They are also called the physics-based models. The physics-based models are also of great significance as it requires both the physical and mathematical insight into the material but this process is complex. It requires a lot of significant research to put the model together. To build such physics-based model, various fundamental information which consist of the structural configurations of a diverse range of food materials, their various properties and their relationship with moisture content and temperature distribution are required. In addition, the optimization of process parameters including drying time level, level of water, drying air velocity is necessary to control the drying operations for better product quality. To obtain this information researchers have worked over the years to develop a smart drying technology (11, 12) but the most common methods are the experimental and empirical methods.

The experimental method for instance is really time consuming, costly, use complex approach and requires a high level of expensive and sophisticated facilities and equipment’s (8) and also it is quite infeasible to perform a series of experiment for a large range of food products.

The physics-based models although very important for the analysis of drying process are also limited in the aspect of prediction of food kinetics and in most cases are unsuitable for real time process control and optimization in drying. They are also computationally time-consuming [29]. This makes this method unsuitable for keeping-up with the present-day consumers’ needs and trends

With the complexity of this problem which involves massive combinatorial spaces or non-linear process and not easily subjective to traditional approaches for a fast and reliable solution, Machine Learning techniques will help to play an important role in solving the problem

Due to the challenges faced by the mathematical models the Machine learning predictive approach has been introduced in food processing to overcome such issues. This Machine learning approach is a very simple and robust method which can be used for future modelling and also capture the drying kinetics of any material of interest. This approach offers us no destructiveness, simplicity and real-time monitoring, as a result of this the Machine Learning  approaches are increasingly under consideration by most food engineers and scientists for application as a superior tool to model complex, dynamic, highly nonlinear, and ill-defined scientific and engineering problems associated with food drying (13).

1.3 PROPOSED SOLUTION

Machine learning which is a predictive based approach helps us to overcome all this challenges and meet up with the present-day trend. In situations when the complexity of a problem is high such that it is nonlinear or involves massive combinatorial spaces and traditional approaches can give a well proven solution, Machine learning approach techniques as proven to be very useful (14-16).

Machine learning is a branch of artificial intelligence which is based on the idea that systems can learn from data (historical kinetic data), identify patterns, develop physical relationship and make future predictions on the food kinetics with minimal human intervention. This approach can be applied to the drying process for the improvement of product quality and overall energy efficiency.

The Machine Learning based approach such as the neural network is an efficient and excellent platform for biological predictive modelling which can characterize historical kinetic data and predict the future food kinetic based on some specific input parameters or training.

1.4 AIM

The aim of this project is to develop a machine learning model that can be used to predict the drying kinetics of food samples under various drying condition in order to fasten new powder product development and formulation.

1.5 OBJECTIVES

The primary objectives of this project are:

- To develop a machine learning model for the prediction of droplets   drying kinetics which include the temperature, mass and size profile, for individual food samples.

- To develop a machine learning model for the prediction of droplets drying kinetics which include the temperature mass and size profile, for a combination of food samples.

1.6 SOLUTION APPROACH

During the research enough experimental data about the substance in consideration is first found and gathered. Then using python codes and different machine learning algorithms a prediction system is built and then trained with the experimental data for each substance. The resulting systems should be able to predict accurate Temperature, Mass and Size values with their individual profiles based on some inputs parameters in which the system must have been trained with for new product developments.

1.7 SIGNIFICANCE OF THE PROJECT

This project aims to solve the limitation of most of the traditional drying technique by giving a fast and reliable Solution. This will help to cut down the time and cost put into the development of new powdery food products with this food. Machine learning algorithms are able to predict the drying properties of material based on the specific input parameters of training with this, food consumer will now be able to demand for specific food products. Machine learning model of drying also presents a non-destructive and simple way for the drying process. It also allows real time monitoring. This approach also provides a superior and powerful tool to model complex, highly nonlinear, dynamic and ill-defined problems associated with drying (13).

CHAPTER TWO

LITERATURE REVIEW

Machine Learning is a branch of Artificial Intelligence that provides systems the ability to learn from data, identify patterns and make important decisions with minimal human intervention. It deals with algorithms and mathematical models that enables a machine (computer) to carry out certain activities without explicit instructions. This field enables computers to learn from past experiences and modify their algorithms to improve its efficiency in it assigned task (17).

The Machine Learning system contains important algorithms that help to control the process of storing, reading and feeding it to a model for learning, i.e., enhancing the performance of the underlying model to solve the given problem.(17).This machine Learning Algorithm’s is a kind of mathematical expression of a problem which help to help to characterize the various linear and nonlinear data. This algorithm is able to produce predictive values based on some input data (8). It also has the superior capacity to handle large amount of data and predict the specific trends and patterns.

Machine Learning consist of different types (17) which includes the Supervised Learning, Unsupervised Learning, semi supervised learning, reinforcement leaning, representation learning etc.

2.1 SUPERVISED LEARNING

The supervised learning involves using model to learn a mapping between input examples and target variable (18). In this type of machine learning a model is fitted on training data which comprises of both input and output .The model is then used to make prediction on test data  in which the only the inputs are feed in and the output from this model are compared to the original withheld target variables used to estimate the performance of the model.

The model improvement for this system always requires an external supervision (17) and the underlying mathematical problems are of regression and classification types.  Classification and regression are the two main types of supervised learning.

Classification: A supervised learning problem that involves predicting a class label

Regression: A supervised learning problem that involves predicting a numeric label

Some of the common algorithms for supervised learning include the linear and nonlinear regression, support vector machine, k-nearest neighbors etc.

2.2 UNSUPERVISED LEARNING

Unsupervised learning is a type of machine learning that uses a model to describe or extract relationships in data (18). The models under unsupervised learning investigates for previously

undetected patterns in a data set with no preexisting labels and with a minimum of human supervision (8). The unsupervised learning operates only on the inputs data without any outputs or target variables. The machine is preprepared and trained on un labeled (or unclassified) data without any form mapping and the algorithm attempts to understand the relationship pattern and discover the outputs (17). Due to this, the unsupervised learning does not have a teacher for correction as of the case of the supervised models.

There are different types of unsupervised learning, the two most common types are:

Clustering: This is the type of unsupervised leaning that involves finding groups in data. In this problem a distance function is used to construct a dissimilarity matrix (19). An example of the clustering Algorithm is the K-Means.

Density estimation. Unsupervised learning that involves giving a summary for the distribution of data

Some additional unsupervised methods are the visualization which involves the plotting or graphing of data in different ways and projection method which as to do with the reduction in dimension of the data (17).

Visualization: Learning problem that involves creating plots of data. An example of a visualization technique would be a scatter plot matrix that creates one very scatter plot of each pair of variables in the data set.

Projection: Learning problem that involves the creation of lower-dimensional representation of data.

Unsupervised learning is mostly used of industry for identifying the critical process parameters, retrieving information, visualizing the data (real-time or offline) density estimation, outlier detection, process modelling etc. (16).

Some of the common algorithms for unsupervised learning includes K-means (20) and C means clustering (21, 22)

2.3 SEMI SUPERVISED LEARNING

Semi supervised learning involves training a Model with partly labeled data and unlabeled data.(17). The main aim of this type of learning is to make an effective use of all the data, not just the labelled data like in supervised learning. This effective use may involve the using of inspiration from unsupervised method such as clustering and density estimation. After the discovery of patterns and groups by the unsupervised learning method then the methods from supervised learning are then used to label the unlabeled examples or apply various labels to unlabeled representation later for prediction.(18).

The model built in this semi supervised learning is used to predicts labels from unlabeled data. This process is also known as “pseudo-labelling” (23)

This type of machine learning is suited when there is huge amount of data and labelling the entire dataset is practically not feasible. A practical example is in the process industry where there is limited quality information about the data of a product which have been accumulated over years. The semi supervised learning will allow the construction of a regression model based on less quality data point in addition to a large data of other process/product variables thereby creating a well performing model (17).

2.4 REINFORCEMENT LEARNING

The supervised learning and unsupervised learning methods always  require that historical data is provided for the modelling task, but the reinforcement methods do not require a form of priori data (17). It is a form of optimal adaptive control problem  that improves upon itself and learns from various  new situation using the trial and error method (24, 25).This type of learning take inspiration from how we human beings learn from data in our various lives (24).The reinforcement learning methods can  also be categorized as semi-supervised model that rely on the action of an unknown algorithm(agent) with an interpreter and reward system in an unknown  environment with  and also its results .For every given iteration of this algorithm the corresponding output result is given to the interpreter to decide maybe the outcome is  favorable or not  . The interpreter then reinforces the solution by providing a reward to the algorithm in situations of the algorithm finding the correct solutions. If not, so the algorithm reiterates until a better result is found.

Some of the examples of reinforcement algorithms are Markov-Decision (26), State-action-reward -action (SARSA) (27).

2.5 REPRESENTATION LEARNING

This is a type of machine learning that relies on the techniques that allow a computer to automatically discover the representations needed for detection or classification from raw data (28). An example of the representation learning is deep learning models based on Artificial neural networks (ANN). Representation learning can be either supervised or unsupervised. As a result of ANNs been used for modeling, they have now been used as a basic tool in the process industry for fault -classification, process monitoring and soft sensor modeling’s (16, 29).

2.6 MACHINE LEARNINGN MODELS

A machine learning model is a computer program that has been trained to recognize some certain patterns and also carryout predictions. The machine learning models are majorly for monitoring or predictions for example processes in the industrial context (17). Monitoring in this sense may involve making analysis of historical data and prediction which would involve analyzing process event that are yet to occur. Machine learning models are based on different types of machine learning and as such the models are categorized into the different learning types as follows:

2.6.1 Supervised Learning Models

Regression

Regression models is a statistical approach that involves identifying a mathematical relationship between a quantity that is continuously varying and output variable (17). Regression analysis enables us see how the dependent variable changes in relation to the independent variable while the other independent are kept constant (30).

The main goal of regression is to predict the future course of gradual development of the output variable as input variables are been changed. An example of a regression problem is predicting the price of an aero plane ticket. Some of the common regression models are as follows (30)

- Linear Regression – This is the most basic regression model; it works best when data can be linearly separable and there is quite a few or no multicollinearity.

- Support vector regression (SVR) – This is based on the same concepts as the Support Vector Machine (SVM) which is mainly used for classification, with a few small modifications suited to handle regression problems.

- Ensemble regression-The aim of this regression is to increase prediction accuracy in learning situations with a numerical target variable by the combination of many models.

- Lasso Regression – This is a linear regression with L2 regularization.

- Ridge Regression – This is a   linear regression with L1 regularization.

2.6.2 Classification

When output variable is a categorical quantity belonging to one of the pre-assigned classes the classification is best suited to this problem (e.g., low, medium and high; yes or no; pass or ail etc.) (17). This is predictive modeling activity where a class label is predated for a given set of input data.

In this case the training data set will be used for the purpose of finding the optimum method to map samples of input data to specified class labels.  Due to this the training set has to sufficiently represent the issue in context and contain a large sizeable number of samples of each class label in order for the model been trained to work efficiently. Some examples of popular classification models:

- Logistic regression – This is a linear model that is the go-to method for binary classification problems.

- The K-Nearest Neighbors algorithm- It is a simple yet time-consuming model that can both be used for classification and regression, KNN classifies a new data point based on similarity. It assumes that similar things exist in close proximity.

- Decision Tree: This is a prediction modeling technique that uses a tree like representation or design and some decision models to make accurate predictions. It is a flow chart -like structure in which each the internal node represents a test on a feature, the leaf node represents a class label and branches represent conjunction of features that lead to those class labels (31).

- Support vector machines: This may be used to classify binary and multiclass data.

- Naive Bayes-This model is based on the Bayesian model.

2.7 UNSUPERVISED LEARNING MODELS

2.7.1 Clustering

Clustering is the process whereby a population or set of data points is portioned into several groups so that the data point in the same group are closely related and different to data point which are in other groups. It can be regarded to as the grouping of certain items based on both their similarities and dissimilarities. The main objective of clustering is to assign the output variables into different clusters based on certain statistical metrics computed by an algorithm (17). Clustering models determines the inherent grouping among unlabeled data. They make certain assumptions about a point similarity and each assumption result in different equally cluster. There is no requirement for a successful clustering but it depends on the user to determine what criteria that will suit their needs (30). Most of the commonly used clustering models are as follows:

- K means - This is a simple clustering model its algorithm tries to partion the dataset into distinct Kpre-defined subgroup that are non-overlapping where reach data point belongs to only one group.

- K means++ - It is a modified version of the K means algorithm. This algorithm solves the drawback of K means algorithm which is that the K-means algorithm is to sensitive to the initialization of the centroids. This algorithm ensures that there is a smarter initialization of centroids and therefore improves the quality of clustering.

- Agglomerative clustering – It is a hierarchical clustering model (bottom-up approach)

2.8 ARTIFICIAL NEURAL NETWORK

As the name implies it is a part of Artificial intelligence that tends to simulate the function of the brain. This approach tends to solve problems that might prove impossible or difficult by human or statistical standards. One of the great capabilities of the Artificial Neural Network is that it can learn by itself which enables them to produce better results as more data becomes available.

Artificial neural networks are built similarly to the human brain, with neuron nodes interconnected like a web. Every neuron is made up of a cell body with the responsibility of processing information by carrying information towards (input) and away (outputs) from the brain.

The ANN framework consists of the input layer, the hidden layer and the output layer and is commonly specified by the architecture, learning algorithms and neuron models (8). Input signals from the user-defined parameters are feed in the form of a pattern, images, this given inputs are mathematical designated by the notations Xn where n=1,2,3 represents the number of inputs, this shown in figure 2.1. The input feed in is then multiplied by the corresponding weight and all the weighted input are summed up inside the computing unit(neuron).A bias is then combined to produce the output nonzero if not it scales up to the  system’s response if the eventual weighted sum is zero(32).One important function of  the bias is that it always has the weight and the input to it to always be equal to 1.The sum of the weighted function inputs can be in the range of 0 to positive infinity and a certain threshold value is benchmarked to keep the response in the limits of the desired value. The sum of the weighed inputs is then passed through an activation function which can be also be referred to as a transfer function used to obtain the targeted output. The network connection intensity can be regularly adjusted by the error between the desired output and the actual output until the satisfactory input-output relationship is obtained.

Figure 2.1   Artificial Neural Network (8)

2.9 TYPES OF NEURAL NETWORK

2.9.1 Feed forward Neural Network

This a self-explanatory neural network wherein the connections between the nodes do not form a cycle (33). It is the very first and simplest type of neural network which was developed (34). In this network data goes in just one direction-forward-from the input nodes to the intermediates hidden nodes until its arrival in the output nodes. This type of networks does not permit cycles or loops (33). There also exist a Deep forward and shallow Feed forward network. A Deep forward network consists of multiple hidden layers of at least 5 between the inputs and the outputs while a shallow Feedforward consist of about 2 to 4 hidden layers both with numerous neurons.

2.9.2 Convolution Neural Network (Convent)

The Convolutional neural network is another class of the Artificial neural network which applies different version of multilayer perceptron concepts as several layers with it are completely interconnected. The main objective of this neural network is to use the learning layers to interpret or detect specific features in a given image as such they are mainly used in visual imagery (35). The identification of features is based on its distance to each pixel (which makes up every facial feature) from a reference point.

2.9.3 Recurrent Neural Networks (RNN)

Recurrent Neural Network is an aspect under neural network that are mostly suited to deal with time series data and other sequential data. As the names implies it involves the recurrence of an operation in the form of loops to facilitate reinforcement learning. It is a type of Neural Network were both the input and outputs are dependent on each other, the output from previous steps is feed in as input into the current step. While other deep neural network assume that inputs and outputs are independent of each other, the output of a recurrent neural network depends on the existing elements within a sequence. One of the distinguishing characteristics of recurrent network is that it shares parameters across each layer of the network. Other feed forward networks have different weights across each node but the recurrent neural network share unique weight parameter within each layer of the network although this weight can still be adjusted through the process of back propagation and gradient descent.

Conventional Neural Network are difficult to train and suffer from problem called the short-term memory (36, 37). If there is a sequence that is long enough, they usually find it difficult to carry information from earlier time steps to later ones. They also suffer from vanishing gradient problem during back propagation. Gradient can be referred to as values used to update the weight in a neural network while the vanishing gradient problem is when the gradient shrinks as it back propagates through time. When a gradient value becomes extremely small it doesn’t contribute to the learning and so the layers that get this small weight stops learning. Those are mostly referred to as the earlier layers. As a result of these layers not learning they forget what is seen in longer sequence those having a short-term memory

New Weight=Weight_Learning rate × gradient (Gradient Update Rule)

Long Short-Term Memory (LSTM)

LSTM is an artificial recurrent neural network architecture which was created as a solution to the short memory faced by the conventional recurrent neural network (37, 38). The Conventional Recurrent Neural network training algorithm need back propagation through time which has vanishing gradient problem. LSTM uses several gates to avoid this problem (38). This gate is the internal mechanism of the LSTM which help to regulate the flow of information. The gates in LSTM have the capability to learn which data in sequence is important to keep or throw away. As a result of this it can pass down relevant information to the long chain of sequence to make relevant predictions. LSTM’s can be found in speech recognition, natural language processing ad sequence prediction (39)

Non-Linear systems modelling with the Recurrent Neural Network (LSTM)

In term of efficiency, we can say that the traditional neural network does not come close to the LSTM, as the LSTM is an extremely efficient tool when the data is sequential. The basic rule of the LSTM is based on the principle that all inputs and outputs are independent of each other. A
dynamic nonlinear system is of the form.

yK=f[yk-1,…yk-ny,uk,…u(k-nu)]      (1)

Where f(.) is an unknown-linear difference equation representing the drying kinetics,

uk is the measurable scalar input

yk is the measurable scalar output.

ny   is the last version of the output.

nu   is the last version of the input.

The discrete-time system as shown in (1) can therefore be identified by the following two types of models (38).

- Simulation model

yK=G[uk,…u(k-nu)]              (2)

Where yK is the output of the model

N[.] is the model structure

yk is the measurable scalar output.

n   is the regression order for the input  yk

Since nu is unknown, n ≠nu.In general form (2) can be written as

yK=N[yk-1,…yk-m,uk,…u(k-nu)]   (3)

where m is the regression order for the model output yK,m ≠ ny.This is a parallel identification model (40).

- Prediction model,

yK=N[yk-1,…yk-m,uk,…u(k-n)]   (4)

where m is regression order for the output yK ,m ≠ ny.This is the series-parallel identification model (41)

N[.] is the model structure

yk is the measurable scalar output.

n   is the regression order for the input  yk

2.10 Related works

In a study by Bai,Xia,Ma &zhou  in 2011 an Artificial neural network(ANN)  modelling was used to predict the moisture ratio and color parameters of Ginkgo biloba (41). With the Levenberg-Marquardt Algorithm as the training function and tansig-purelin as the network transfer function, the ANN model was trained for finite iteration calculations. According to findings the results showed that the ANN algorithm could accurately predict experimental data with a high correlation coefficient (0.9056–0.9834) and a low mean square error (0.0014–2.2044). Also, the developed ANN models can be used to predict moisture content and color changes in ginkgo biloba seeds while they are being microwave dried.

The multilayer "feed-forward" ANN structure utilized in this study (Figure 2.2) consisted of one input layer, one hidden layer, and one output layer with a convergence criterion for training purposes. Microwave power and drying time were the input variables in the input layer, whereas moisture content and color parameters of GB seeds at any moment were the output variables in the output layer. After a period of trial and error, a network unit with the hyperbolic tangent sigmoid transfer function "tansig" for hidden layer neurons, 10 neurons in the hidden layer, linear transfer function "purelin" for output layer neurons, and Levenberg-Marquardt training algorithm "trainlm" for the training function was chosen.

Figure 2.2 Artificial neural network configuration (41)

In 2012 Momenzadeh, Zomorodian, Mowla studied the application of the Artifical Neural Network for predicting the drying time (output parameter) (42). The model's input parameters were microwave power, drying air temperature, and green pea moisture content. To investigate the effects of transfer functions and training techniques, a 50-neuron ANN model was chosen. The results indicated that for the green pea drying system, a network with the logsig (Log sigmoid) transfer function and trainrp (Resilient back propagation; Rprop) back propagation algorithm generated the best accurate predictions. The root mean square error (RMSE), mean absolute error (MAE), and standard error (SE) were calculated to evaluate the ANN model, and the results indicated that the random errors were within an acceptable range of ±5% with a coefficient of determination (R2) of 98 %.

Figure 2.3 Selected artificial neural network structure.

Motavali, Najafi, Abbasi, Minaei, & Ghaderi in 2013 investigated the use of a feedforward Artificial Neural Networks (ANN) trained by standardard back propagation algoithms(43) .It was used to study the effects of microwave power, pressure and drying time on moisture ratio (MR) and drying rate (DR) based on the evaluation of experimental data fitting with semi-theoretical and empirical models. The hidden layer's activation function was set to logistic, whereas the output layer's was set to tangent. A variety of different networks with various purposes and topologies were investigated. The ANN model was able to predict the moisture ratio and drying rate fairly well with  determination coefficients (R2) of 0.9996, 0.9961, and 0.9958 for training, validation, and testing,  For training, validation, and testing, the prediction Mean Square Error of ANN was around 0.0003, 0.0071, and 0.0053, respectively.

Figure 2.4 . Configuration of multilayer neural network for predicting MR (Moisture Ratio) and       DR (Drying Rate)

CHAPTER THREE

METHODOLOGY

3.1 DATA COLLECTION

Obtaining data is the initial step in any Machine Learning process. This procedure is dependent on the scope of the project and the sort of data been working with. Our model's accuracy is determined by the quantity and quality of your data. For this project we were able to collate the kinetic drying data for three different food components which are Lactose, Fat, Protein and two other food materials which are whole milk and skim milk. These data are obtained through experimental methods in the Laboratory.

The collated data consist of lactose data of 10% and 20% composition, fat of 30% composition protein of 9.3% and 30% composition, whole milk of 20% composition and skim milk of 20% composition. It also contains:

- Temperature of Air varying from 65°C-110°C

- Initial droplet size varying from 1 μL-4.19μL.

- Constant Airflow velocity varying from 0.45m/s-0.75m/s

- Constant relative humidity varying from 0.01 kg/kg-0.02 kg/kg.

- Time

- Temperature of droplets at different times.

- Mass of droplets at different times

3.2 DATA PREPARATION

Unorganized, missing, or noisy components are common in real-world or experimental data. As a result, after selecting the data, we must clean, prepare, and alter it in order to achieve Machine Learning success.

This is a crucial part in the process, and people often spend up to 80% of their time here. Having a clean data collection improves the accuracy of your model in the long run. This process for our project involved:

- Cleaning the data (remove duplicates, correct errors, deal with missing values, normalization, data type conversions, etc.)

- Randomize data to remove the impacts of the sequence in which we gathered and/or otherwise prepared our data.

- Perform further exploratory analysis, such as visualizing data to assist discover important correlations between variables or class imbalances (bias alert!).

- Splitting data into training and testing sets.

3.4 DATA NORMALIZATION

Normalization is a data preparation method that is frequently used in machine learning. It is the process of converting the values of numeric columns in a dataset to a similar scale without distorting the ranges of values.

A data set may be fantastic with a lot of interesting features, but if you don't normalize it, one of them will absolutely dominate the others and this has effect on the accuracy of the model.

Variables assessed at various scales may not contribute equally to the model fitting and learnt function, which might lead to bias which data normalization helps to deal with this potential problem feature wise (44)

Machine learning algorithms tend to perform better when numeric input variables are scaled to a standard range. The Data Normalization technique employed for this project is the Min-Max Scaler.

The Min-Max scaler helps to translate and transform the features such that all features will be converted into the range [0,1], with the minimum and maximum values of each feature/variable being 0 and 1, respectively.

This normalization technique is very useful because when input features are min-max scaled (or in general scaled), back-propagation can be more stable and even faster than when the data is unscaled. This can be very useful for some ML models, such as Multi-layer Perceptron (MLP), LSTM where the back-propagation can be more stable and even faster when input features are min-max scaled (or in general scaled).

The algorithm behind the Min-Max Scaler is as follows:

- For each feature X, we calculate the max value (Xmax) and min value (Xmin)(in absolute terms)

- For each value in that feature X (Xi), calculate:

Xscaled= Xi-XminXmax-Xmin

For this project the default Min-Max scaler configurations from sklearn.preprocessing has been used. First, a Min-Max scaler instance is defined with preferred default parameters. When done, we now call the fit transform () function and pass in our dataset to create a transformed version of our dataset.

3.5 CREATION OF TRAINING AND TESTING SET OF DATA

The data collated is split into a training portion and a test portion, so that our model can be trained with the training portion and then tested with the testing data. It always a good idea to partition the data in such a manner that the model can be trained on a greater piece of it, allowing it to adapt to a wider range of data configurations. The procedure adopted for this project is that we split the data into two data sets which are the training data and test data.

TRAINING DATA

The training data for this project is used for the model fitting, it includes:

- The whole of the Protein data

- The whole of the Fat data

The whole of the lactose data except the Lactose data with temperature of air at 110°C and Initial droplet size at 3 μL

Table 3.1 Training data set

| Food Material | Temperature of Air(°C) | Composition (%) | Volume (μL) | Velocity (m/s) | Relative Humidity(kg/kg) | Time (s) |
| --- | --- | --- | --- | --- | --- | --- |
| Lactose | 67.5-110 | 10 ,20 | 1 -3 | 0.45,0.75 | 0.01 | 0-600 |
| Fat | 67.5-110 | 30 | 1.6 | 0.45 | 0.01 | 0-600 |
| Whey Protein | 65-80 | 9.3,30 | 1.6,4.19 | 0.45,0.5 | 0.01,0.02 | 0-600 |

Table 3.1 shows in   detail the independent variables for our training data set

TESTING DATA

The test data for this project is used for estimating the model’s accuracy, it includes:

- Lactose data with temperature of air at 110°C and Initial droplet size at 3 μL.

- The whole of the whole milk data

- The whole of the skim milk data.

Table 3.2 Testing data set

| Food Material | Temperature of Air(°C) | Composition (%) | Volume (μL) | Velocity (m/s) | Relative Humidity(kg/kg) | Time (s) |
| --- | --- | --- | --- | --- | --- | --- |
| Lactose | 110 | 10 | 3 | 0.75 | 0.01 | 0-300 |
| Whole milk | 67.5-106.6 | 20 | 1.59 | 0.45 | 0.01 | 0-300 |
| Skim milk | 67.5-106.6 | 20 | 1.59 | 0.45 | 0.01 | 0-300 |

Table 3.2 shows in detail the independent variables for our testing data set

3.6 SEEDING

In Artificial Neural Networks (ANN) there is said to be randomness when the same Artificial Neural Network (ANN) is trained on the same data and its produces different results.

This is because the weights for our model are initially initialized with random variables when we train it. As a result of the random initialization, the weights will start off with distinct random values when we train the model, and will then change dynamically during training via gradient descent. This will be the case for each time we train. So, for each of these distinct periods, we'll start with a new set of random weight values.

Randomness in ANN can be caused by a variety of factors (45) which include:

- Random initialization of weights and biases

- Randomness in regularization techniques like Dropouts

- Randomness in optimization techniques like Stochastic Gradient Descent (SGD)

By design, neural networks employ randomness to guarantee that they learn the function being approximated for the problem efficiently but there are instances when we want our model to provide the same results regardless of when we train it, as long as we use the same training data. This can be done through seeding.

Seeding helps to get predictable, repeatable result every time we train our model on the same data. Setting the seed to a specific value, such as 0 or 123, will result in the same random numbers being generated over and over again on the same or other machines.

To achieve reproducible results this for this ANN project we use

- NumPy random seed

- Tensor Flow set_random_seed

The NumPy library and tensor flow library is imported into the file and then seed are set to a specific value.

Figure 3.1 Code snippet for the Seeding process

3.7 MODEL

The nature of our data set consists of independent variables and continuous dependent variable which is a machine learning regression problem and so a regression model can be applied to the dataset to predict a continuous dependent variable from a number of independent variables.

The LSTM model for regression was used to solve the machine learning problem for this project. In Machine Learning, LSTM for regression is often a time series problem. The key distinction between time series and other machine learning problem is that the data samples are in sequence. Whether overtly or indirectly, the sequence indicates a time dimension. The timesteps of the input sequence are the implicit component (46).

LSTM MODEL

Long Short-Term Memory (LSTM), a model initially proposed in 1997 (46) is type of Recurrent Neural Network in Deep Learning that has been specifically developed for the use of handling sequential prediction problems. The LSTM network uses LSTM cell blocks instead of traditional neural network layers. The input gate, forget gate, and output gate are three components of these cells. The LSTM cell is represented graphically as follows:

Figure 3.2 LSTM CELL DIAGRAM (47)

From figure 3.2. Our sequence value $x_t$is concatenated with the preceding output from the cell $h_{t-1}$  on the left hand side. The first step is squashing this combined input with the use a tanh layer. The input is then routed through an input gate in the second phase. A squashed input is multiplied by the output of an input gate, which is a layer of sigmoid activated nodes. An input gate is a layer of sigmoid activated nodes whose output is to be multiplied by the squashed input. Any elements of the input vector that aren't required can be "killed" by these input gate sigmoids. Sigmoid functions are said to output values between 0 and 1. Therefore a sigmoid function is used to ensure that the weights connecting the input to these nodes can be trained to output values close to zero to “switch off” certain input values (or, conversely, outputs close to 1 to “pass through” other values).

The internal state or forget gate loop is the next step in the data flow through this cell. The cells in LSTM have an internal state variable $s_t$. This variable, lagged one time step i.e. $s_{t-1}$ is added to the input data to create an effective layer of recurrence. This addition operation is used rather than a multiplication operation because it reduces the risk of vanishing gradients. This recurrence loop, however, is controlled by a forget gate, which functions similarly to an input gate but assists the network in learning which state variables should be "remembered" or "forgotten."

The final steps involve a tanh squashing function at the output layer, whose output is controlled by an output gate. This gate decides which values from the cell $h t$ are actually allowed as an output.

An overview of the steps in building an LSTM model in Keras are as follows;

- Create Environment

- Define Network

- Compile Network

- Fit Network

- Evaluate Network

- Make Predictions

3.7.1 Create Environment

This first step in creating an environment for this project involved that Anaconda be installed which is a python (Anaconda is a Python distribution (a prebuilt and preconfigured collection of packages) that is widely used in data science. Anaconda makes it simple to install and update Python, Jupyter Notebook, Jupyter Notebook and other regularly used scientific computing and data science packages. The current version of Python (Python 3.5) was installed via the Anaconda.

The Jupyter notebook from anaconda which is an interactive development environment for Jupyter notebooks, code, and data that is accessible via the web was used specifically for this project. JupyterLab is adaptable which made it easy to customize and arrange the user interface to support a variety of data science, scientific computing, and machine learning workflows (48).

The next step in creating a sustainable environment for the python codes involves the installation of and importation of various python libraries. These libraries are as follows:

- Sckit-learn

- Pandas

- NumPy

- Matplotlib

- Scipy

- Sklearn

- Tensor flow

- Keras with Tensorflow

3.7.2 Define Network

The first step in defining the network is to build a Sequential class instance. This is followed by stacking the layers and arranging them in an orderly manner

By adding LSTM layers to the Sequential model, they can be layered. When stacking LSTM layers, we must output a sequence rather than a single value for each input so that the next LSTM layer may get the appropriate 3D input. By setting the return sequences argument to True we were able to accomplish this in our project. Each layer is followed by a dropout function. Dropout functions are functions that are used to prevent overfitting. They work by randomly setting the outgoing edges of hidden units to specified values.

The next step is to add a dense layer. Dense layer is a fully connected layer that frequently follows LSTM layers and is then used to output a prediction .

It is important to note that the first layer of the network must specify the expected number of inputs. Three-dimensional input, consisting of samples, timesteps, and features, is required:

- Samples. In the data, these are the rows.

- Timesteps. Past observations for a feature, such as lag variables, are stored here.

- Features. These are your data's columns

Also, the input shape of our data was specified at the first layer of our model and also the number of neurons at each layer.

Figure 3.3 Code snippet for the Model Definition

For this this project a for loop was defined to iterate through to some of the network parameters such as;

- Hidden layers: A total of 1- 5 hidden layers were defined.

- Neurons: A range of 1-30 neurons were considered with steps size of 2

- Time steps: A range of 1-61 timesteps were considered with steps size of 10

3.7.3 Compile Network

Compilation can be said to be a productive step. It converts the simple layer sequence we established into a very efficient series of matrix transforms in a format that can be run on your GPU or CPU, depending on how Keras is set up.

The compilation stage requires that a number of parameters must be given during compilation, all of which are individually designed to training the network. The optimization algorithm to use to train the network, as well as the loss function to use to evaluate the network that the optimization algorithm minimizes and metrics to collect while fitting your model in addition to the loss function. The parameters specified for the model compilation are follows;

- For Optimization Algorithm: ‘adam’ used for tuning the learning rate and updating network weight.

- For Loss Function: ‘mean_squared_error’ for regression problem.

- For metrics: ‘accuracy’ specified by the name in an array

Figure 3.4 Code snippet for the Model Compilation

3.7.4 Fit Network

The fitting process of the network requires that the training data to be specified both the matrix of independent variables and an array of continuous dependent variable.

The backpropagation algorithm is used to train the network, which is then optimized using the optimization algorithm and loss function specified when the model was built.

Backpropagation requires the network to be trained for a certain number of epochs or exposures to the training dataset.

The number of epochs is also specified when we want to fit our network. Each epoch may be divided into batches, which are groupings of input-output pattern pairings. The number of patterns that the network is exposed to before the weights are changed throughout an epoch is defined by this parameter. It's also an efficiency improvement, guaranteeing that only a limited number of input patterns are loaded into memory at any given moment.

A total of 1000epochs with size of 32 were used to fit the network model.

3.7.5 Evaluate Network

The network model can be evaluated once the network is done. The model is evaluated using the training data and the R2 score from the sklearn. metrics is used to measure the accuracy.

The R2 score also known as the Coefficient of determination. It is the degree of variance in the output dependent characteristic that can be predicted based on the input independent variable (s). It's used to assess how effectively the model reproduces observed findings, based on the ratio of total deviation of results represented by the model. It can be mathematically represented as

R2=1-SSresSStot

Where 
SSres  is the sum of squares of the residual errors.

SStot  is the sum of squares of the residual errors.

3.7.6 Make Prediction

Having been satisfied with the model’s performance, the model is then used to carryout predictions. The model predict function was used to do this with an array of fresh input patterns

The test data specified at table3.2 were used to make predictions and were compared to the actual values.

CHAPTER FOUR

RESULTS AND DISCUSSION

4.1 INTRODUCTION

This chapter entails the results and analysis of the LSTM model generated by Python including how the optimized parameters were generalized.

Long Short-Term Memory (LSTM)

For this project the LSTM model was trained with different parameters using a for loop . The parameter which was varied include:

- The number of hidden layers (in range of 1-5)

- Window (in range 1-51)

- No of Neurons (in range 2-28)

The trained network was also tested with some food materials like Lactose, skim milk and whole milk. The purpose of varying this parameter is to determine the best optimum conditions for the predicting the drying kinetics (Temperature profile and mass profile) of the food material. The performance metrics consider was the R2 Score

It was determined that best optimum parameters which can used for evaluating the Lactose food material was when the window is 11, Number of hidden layers is 2 and the Number of neurons is 18.

For the whole milk food composition which contains lactose, protein, fat etc., the network with the window of 1, 1 hidden layer and 6 neurons is used to   evaluate the kinetics of the food material

Also, for the Skim milk food composition which contains lactose protein fat etc., the network with the window of 1, 1 hidden layer and 6 neurons is used to evaluate the kinetics of the food material

4.2 EFFECTS OF KEEPING NEURON AND WINDOW CONSTANT AND CHANGING LAYERS

From Figure 4.1

Lactose droplet for 2 Neurons and 1 window

From the plots, it can be observed that as the number of epochs increases the loss also decreases. This was observed for layer 1-5. For layer 2,3,4 it took more than 50 epochs in order to reduce the loss while for the for layers 1 and 5 it took a total of 50 epochs. The plots for each layers show that it as a high learning rate.

The R2 Score for the train set is observed to move from 0.952886825 for layer 1 to 0.897453684 for layer 5 which shows that as the number of layers do increase there is a gradual reduction in the R2 Score value. Also, for the Train set, it is observed that there is a gradual reduction in the R2  score  value  as the number of layers increases. The R2 score of 0.948831373 for layer 1 gradually reduces till 0.92106227 for layer 5. This indicates that changing the number of layers as significant effect on the R2 score value

Lactose droplet for 16 Neurons and 1 window

From the plots, it can be observed that as the number of epochs increases the loss also decreases at some points until the curve reaches steady state. This was observed for all layers. The plots for each layers show that it as a high learning rate.

The R2 Score for the train set is observed to be within the range of 0.977-0.978for layers 1-5. Also, for the Test set, it is also observed that the range of R2 score is within the range of 0.995-0.997 for layer 1-5. This indicates that changing the number of layers does not significantly affect the R2 score Value.

Lactose droplet for 28 Neurons and 51 window

From the plots, it can be observed that as the number of epochs increases the loss also decreases. This was observed for all layers. It took the network of layer 1-3 a total of 87 epochs, layer 4 a total of 83 epochs, layer 5 a total of 90 epochs to reduce the loss. The curves for each layers show that it   has a good learning rate.

The R2 Score for the train set is observed to be within the range of 0.981-0.986 for layers 1-5 .Also, for the Train set, it is observed that the range of R2 score is within the range of 0.994-0.999 for layer 1-5. This indicates that changing the number of layers does not significantly affect the R2 score value.

Lactose (Layer1-5)

From the plots, it is observed that when the network is  of 2 neurons and a window 1 the R2 Score is dispersed within the range 0.8 and 1 which shows that the effect of layers 1-5 on the network is significant. The network of neuron 16 and window 1 the R2 Score is seen to overlap which shows that changing the layers of the network within the range of 1-5 does not really affect the R2 Score for the network. Also, for the network of neuron 16 and window 51 the R2 Score is seen to overlap which shows that changing the layers of the network within the range of 1-5 does not really affect the R2 Score for the network.

Figure 4.1 Effects of keeping neuron and window constant and changing layers                           (10wt%, v = 0.75m/s, H=0.001kg/kg)

4.3 EFFECTS OF KEEPING LAYERS AND NEURONS CONSTANT AND CHANGING THE WINDOW SIZE

From Figure 4.2

Lactose droplet for 2 neurons and 1 hidden layer

From the plots, it can be observed that as the number of epochs increases the loss also decreases until it reaches a steady state. This was observed for windows 1-51. It took the network with window of 1 a total of 47 epochs, window of 11 a total of 51 epochs, window of 21 a total of 50 epochs, window of 31 a total of 44 epochs, window of 41 a total of 32 epochs, window of 51 a total of 69 epochs to reduce the loss. For the network with 1,21,31,41,51 window the curve generated shows that it has a high learning rate.

The R2 Score for the train set is observed to take different values within the range of 0.929 – 0.949 for the network with 1-51 windows. Also, for the Test set, it is observed that the R2 Score takes different values within the range of 0.952-0.977. This indicates that changing the windows size has significant effect on the R2 score value.

Lactose droplet for 16 neurons and 1 hidden layer

From the plots, it can be observed that as the number of epochs increases the loss also decreases until it reaches a steady state. This was observed for windows 1-51. It took the network with window of 1 a total of 109 epochs, window of 11 a total of 44 epochs, window of 21 a total of 100 epochs, window of 31 a total of 120 epochs, window of 41 a total of 116 epochs, window of 51 a total of 46 epochs to reduce the loss. For the network with 1,21,31,41,51 window the curve generated shows that it as a high learning rate.

The R2 Score for the train set is observed to increase from 0.978499152 for the network with 1 window to 0.983896971 for the network with 11 window it then stays within the range of   0.980-0.986 for the network with 21,31,41,51 window which shows that increasing the window size from 1 to any other window size increases the R2 Score value. Also, for the Test set, it is observed that R2   score of   0.997240519 for network with 1 window size increases to 0.998103511 for the network with 11 window size it then stays within the range of 0.995-0.999 for the network with 21,31,41,51 window size. This indicates that increasing the window size from 1 to any other window size increases the R2 Score value.

Lactose droplet for 28 neurons and 5 hidden layers

From the plots, it can be observed that as the number of epochs increases the loss also decreases until it reaches a steady state. This was observed for windows 1-51. It took the network with window of 1 a total of 53 epochs, window of 11 a total of 63 epochs, window of 21 a total of 104 epochs, window of 31 a total of 78 epochs, window of 41 a total of 87 epochs, window of 51 a total of 91 epochs to reduce the loss. For the network with 1,21,31,41,51 window the curve generated shows that it has a high learning rate.

The R2 Score for the train set is observed to take different values within the range of 0.929 – 0.949 for the network with 1-51 windows. This indicates that changing the windows size has significant effect on the R2 score value.

Lactose (Window 1-51)

From the plots, it is observed that when the network is of 2 neurons and of layer 1  the R2 Score is dispersed which indicates that changing the window size as relative effect on the R2 score. For the network of   16 neurons and 1 layer the R2 Score is seen to overlap each other apart from the network with 1 window size this indicates that changing the window size as relative effect on the R2 score

Also, for the network of 28 neurons and of layer 5 the R2 Score is dispersed within the range of 0.951. This indicates that the effect of changing the number of windows is significant on the R2 score value.

Figure 4.2 Effects of keeping layers and neurons constant and changing the window size                     (10wt%, v = 0.75m/s, H=0.001kg/kg)

4.4 EFFECTS OF KEEPING LAYERS AND WINDOW CONSTANT AND CHANGING NEURONS

From Figure 4.3

Lactose droplet for 1 Layers and 1 Window

From the plots, it can be observed that as the number of epochs increases the loss also decreases. This was observed for Neurons 2-28. It took the network of 2 neurons a total of 47 epochs, 8 neurons a total of 70 epochs, 16 neuron a total of 109 epochs, 24 neuron a total of 54 epochs,28 neuron a total of 57 epochs to reduce the loss. For the network with 2,8,16,24,28 neuron the curve generated shows that it as a high learning rate.

The R2 Score for the train set is observed to increase from 0.948831373 for network with 2 neurons to 0.97545301 for the network with 8 neurons it then stays within the range of   0.975-0.978 for the network of 16,24,28 neurons which shows that increasing the number of neurons from 2 to any other number of neurons increases the R2 Score value. Also, for the Test set, it is observed that R2  Score of   0.952886825 for network with 2 neurons  increase to 0.97545301 for the network with 8 neurons it then stays within the range of   0.994-0.998  for the network of 16,24,28 neurons .This indicates that changing the number of neurons form 2 neurons to any other neuron does significantly affect the R2 score value .

Lactose droplet for 2 Layers and 11 Window

From the plots, it can be observed that as the number of epochs increases the loss also decreases. This was observed for Neurons 2-28. It took the network of 2 neurons a total of 53 epochs, 8 neurons a total of 77 epochs, 16 neuron a total of 68 epochs, 24 neuron a total of 64 epochs,28 neuron a total of 43 epochs to reduce the loss. For the network with 2,8,16,24,28 neuron the plots generated shows that it as a high learning rate.

The R2 Score for the train set is observed to move from 0.942230309 for the network with 2 neurons to 0.980550374 for the network with 8 neurons it then stays within the range of   0.981-0.987 for the network of 16,24,28 neurons which shows that increasing the number of neurons from 2 to any other number of neurons increases the R2 Score value. Also, for the Test set, it is observed that R2  Score of   0.931388131 for network with 2 neurons  increase to 0.994602559 for the network with 8 neurons it then stays within the range of   0.995-0.999  for the network of 16,24,28 neurons .This indicates that changing the number of neurons from  2 neurons  to any other neuron does affect the R2 score significantly

Lactose droplet for 5 Layers and 55 Window

From the plots, it can be observed that as the number of epochs increases the loss also decreases. This was observed for Neurons 2-28. It took the network of 2 neurons a total of 56 epochs, 8 neurons a total of 67 epochs, 16 neuron a total of 100 epochs, 24 neuron a total of 85 epochs,28 neuron a total of 90 epochs to reduce the loss. For the network with 8,16,24,28 neuron the curve generated shows that it as a high learning rate and for the network with 2 neuron the curve generated shows that it as a good learning rate.

The R2 Score for the train set is observed to increase as the number of neurons increase Also, for the Test set, it is observed that R2  score of   0.980299113 for network with 2 neurons reduces to 0.972700118 for the network with 8 neurons it then increases   to 0.995387317  for the network  with 16 neurons and stays within the range of   0.995-0.999  for the network of 24,28 neurons .This indicates that changing the number of neurons  does significantly affect the R2 score value .

Lactose (Neuron 2-28)

From the plots, it is observed that when the network is of 1 layer and a window 1 is taken the R2 Score is dispersed within the range 0.95 and 1. The R2 Score for the network with neuron 2 is seen to be separate from other R2 score for another network with neuron 8-28 which overlap. For the network of   2 layers and a window 11 the R2 Score for neuron 2 is dispersed from others within the range 0.9 and 1. The R2 Score for the network with neuron 2 is seen to be separate from other R2 scores for other networks with neuron 8-28 which overlap.

This shows that the effect of changing the number of neurons from 2 to  any other neuron within the range of 2-28 as significant effect on the R2 score of the network. Also, for the network of 5 layers and a window 51 the R2 Score is dispersed within the range of 0.9-1. This indicates that the effect of changing the number of neurons is significant on the R2 score value.

Figure 4.3. Effects of keeping layers and window constant and changing neurons                              (10wt%, v = 0.75m/s, H=0.001kg/kg)

4.5 DRYING KINETICS OF LACTOSE

From Figure 4.5. Temperature and Mass profile of Lactose at 110oC

Temperature Profile

It is observed that the predicted values overlap the measured values. The predicted values are observed to follow the same trend as that of the measured values which indicates that the model was efficient enough to predicts values close to measured values. The R2 score for the training is observed to lower than the predicted.  Also, the points for measured vs predicted plots were all observed to fall along a straight diagonal line this indicates that we have a high goodness of fit for our model with high R2 score.

Mass Profile

The predicted values are observed to follow the same trend as that of the measured values as some were seen to overlap this indicates that the network with 11 windows,2Layers and 18 Neuron was efficient enough to predicts values relatively very close to measured values with an R2 score of 0.99937. The R2 score of 0.97303 for the training is observed to lower than the predicted which means that the model is not overfitted.  Also, the points for measured vs predicted plots were all observed to fall along a straight diagonal line this indicates that we have a high goodness of fit for our model with high R2 score.

Figure 4.4. Temperature and Mass profile of Lactose at 110oC (a)Temperature profile (b) Mass Profile (10wt%, v = 0.75m/s, H=0.001kg/kg)

Lactose droplet for 11 window ,2 Layers and 18 Neuron

From Figure 4.6. Temperature and Mass profile of Lactose at 110oC

Temperature Profile

For the Lactose droplet for 1 window,3 Layers and 24 Neuron ,it was observed that the predicted values overlap the measured values. The predicted values are observed to follow the same trend as that of the measured values which indicates that the model was efficient enough to predicts values close to measured values. The R2 score for the training is observed to be lower than the predicted. Also, the points for measured vs predicted plots were all observed to fall along a straight diagonal line, all this indicates that we have a high goodness of fit for our model with high R2 score.

Mass Profile

For this set of parameters, it is observed that the predicted values follow the same trend as that of the measured values which indicates that the model was efficient enough to predicts values close to measured values with an R2 score of 0. 99691. The R2 score for the training is observed to be lower than the predicted which means that there is no overfitting.  Also, the points for measured vs predicted plots were all observed to fall along a straight diagonal line this indicates that we have a high goodness of fit for our model with high R2 score.

(b)`

Figure 4.5. Temperature and Mass profile of Lactose at 110oC (a)Temperature profile (b) Mass Profile (10 wt %, v = 0.75m/s, H=0.001kg/kg)

(a)`

4.5 DRYING KINETICS OF WHOLEMILK

For the prediction of our whole milk drying kinetics a window size of 1 ,1 hidden layer and 6 neurons were considered. This gave us an R2 score   of 0.9 and above which indicates that this model was efficient in the prediction of the drying kinetics of the whole milk as compared to the measured values

From Figure 4.6-4.8 for temperature and mass profile. It can be observed that the predicted values were close to the measured values and some were noticed to overlap. The predicted values are also observed to follow the same trend as that of the measured values which indicates that the model was efficient enough to predicts values close to measured values. Also, for the measured vs predicted plots the data points were all observed to be very close to the straight diagonal line this indicates that we have a high goodness of fit for our model with high R2 score for both the temperature and mass profile.

4.5 DRYING KINETICS OF SKIM MILK

The prediction for skim milk drying kinetics a window size of 1 ,1 hidden layer and 6 neurons was also considered. This also gave us an R2 score   of 0.9 and above which indicates that this model was efficient in the prediction of the drying kinetics of the whole milk as compared to the measured values

From Figure 4.9-4.11 different   drying test cases were considered for temperature and mass profile It can be observed that the predicted values were close to the measured values and some were noticed to overlap. The predicted values are also observed to follow the same trend as that of the measured values which indicates that the model was efficient enough to predicts values close to measured values. Also, for the measured vs predicted plots the data points were all observed to be very close to the straight diagonal line this indicates that we have a high goodness of fit for our model with high R2 score for both the temperature and mass profile.

(b)`

(a)`

Figure 4.6. Temperature and Mass profile of Lactose at 110oC (a)Temperature profile (b) Mass Profile (10 wt %, v = 0.75m/s, H=0.001kg/kg)

Figure 4.7. Temperature and Mass profile of whole milk at 67.5oC (a)Temperature profile (b) Mass Profile (20wt%, v = 0.45m/s, H=0.001kg/kg)

(b)`

(b)`

Figure 4.7. Temperature and Mass profile of whole milk at 87.1oC (a)Temperature profile (b) Mass Profile (20wt%, v = 0.45m/s, H=0.001kg/kg)

(a)`

Figure 4.7. Temperature and Mass profile of whole milk at 67.5oC (a)Temperature profile (b) Mass Profile

(20wt%, v = 0.45m/s, H=0.001kg/kg)

(b)`

(b)`

Figure 4.8. Temperature and Mass profile of whole milk at 106.6oC (a) Temperature profile          (b) Mass Profile (20wt%, v = 0.45m/s, H=0.001kg/kg)

(a)`

Figure 4.9. Temperature and Mass profile of skim milk at 67.5oC (a) Temperature profile (b) Mass Profile (20wt%, v = 0.45m/s, H=0.001kg/kg)

(b)`

(a)`

(b)`

(b)`

(a)`

Figure 4.10. Temperature and Mass profile of skim milk at 87.1oC (a) Temperature profile (b) Mass Profile (20wt%, v = 0.45m/s, H=0.001kg/kg)

Figure 4.11. Temperature and Mass profile of skim milk at 106.6 oC (a) Temperature profile (b) Mass Profile (20wt%, v = 0.45m/s, H=0.001kg/kg)

(b)`

(a)`

CHAPTER FIVE

CONCLUSION

Machine learning has once again been successfully implemented to solve a problem. This is seen in the field of drying. This report has helped to show the efficiency of machine learning.

The findings of this study demonstrate that the proposed LSTM model obtained using the optimization approach are good models for predictions of drying kinetics and should perform well with their predictive power on food powders datasets containing lactose, protein and fat.

Table 5.1 Optimal parameters

| FOOD POWDER | Parameters |
| --- | --- |
| Whole Milk | 1 window,1 Layers,6 Neurons |
| Skim milk | 1 window,1 Layers ,6 Neurons |
| Lactose | 11 window,2 Layers,18 Neurons |

From Table 5.1 The best optimum parameters to get a good prediction has been listed for the food powder considered in this study.  The LSTM network used in this study was also able to give us an R2 score within the range of 0.93-1 which indicates that the artificial recurrent neural network is efficient in the prediction of the drying kinetics for food powder.

REFRENCE

1.	Uwem Inyang  IO, Benjamin Etuk Drying and the Different Techniques. International Journal of Food Nutrition and Safety. 2017 8(1):45-72.

2.	Inyang UE, Oboh IO, Etuk BR. Kinetic Models for Drying Techniques—Food Materials. Advances in Chemical Engineering and Science. 2018;08(02):27-48.

3.	Fateh Singh VKK, and B. P. Singh. Mathematical Modeling to Study Drying Characteristic of Apple and Potato. 2014.

4.	Kabiru A, Joshua AA, Raji A, editors. EFFECT OF SLICE THICKNESS AND TEMPERATURE ON THE DRYING KINETICS OF MANGO (Mangifera Indica)2013.

5.	Shibasish Roy MZR, Tapash Kumar Burman. Effect of different softeners on moisture absorption and transmission properties of knitted cotton fabric: Daffodil Innternational University; 2015.

6.	Yiew Ca. Pharmaceutical Dosage forms: Parenteral Medications. Parenteral Medications Indian Journal of pharmaceutical science and technology1981;35:106 -18.

7.	Chou SK, Chua KJ. New hybrid drying technologies for heat sensitive foodstuffs. Trends in Food Science & Technology. 2001;12(10):359-69.

8.	Khan MIH, Sablani SS, Joardder MUH, Karim MA. Application of machine learning-based approach in food drying: opportunities and challenges. Drying Technology. 2020:1-17.

9.	Freire FCbB, Vieira GNA, Freire JT, Mujumdar AS. Trends in Modeling and Sensing Approaches for Drying Control. Drying Technology. 2014;32(13):1524-32.

10.	Shrithran V. Modelling of a process intensification of drying process using Reaction Engineering Approach(REA).

11.	Su Y, Zhang M, Mujumdar AS. Recent Developments in Smart Drying Technology. Drying Technology. 2015;33(3):260-76.

12.	Vynnycky M. An asymptotic model for the primary drying stage of vial lyophilization. Journal of Engineering Mathematics. 2016;96(1):175-200.

13.	Sun Q, Zhang M, Mujumdar AS. Recent developments of artificial intelligence in drying of fresh food: A review. Critical Reviews in Food Science and Nutrition. 2019;59(14):2258-75.

14.	Butler KT, Davies DW, Cartwright H, Isayev O, Walsh A. Machine learning for molecular and materials science. Nature. 2018;559(7715):547-55.

15.	Kadlec P, Gabrys B, Strandt S. Data-driven soft sensors in the process industry. Computers & chemical engineering. 2009;33(4):795-814.

16.	Ge Z, Song Z, Ding SX, Huang B. Data Mining and Analytics in the Process Industry: The Role of Machine Learning. IEEE Access. 2017;5:20590-616.

17.	Martynenko A, Misra NN. Machine learning in drying. Drying Technology. 2019;38(5-6):596-609.

18.	Brownlee J. 14 Different Types of Learning in Machine Learning 2019 [

19.	Misra NN, Carl S, Cullen PJ. Process Analytical Technology (PAT) and Multivariate Methods for Downstream Processes. Current Biochemical Engineering. 2015;2(1):4-16.

20.	Lloyd S. Least squares quantization in PCM. IEEE Transactions on Information Theory. 1982;28(2):129-37.

21.	Bezdek JC, Keller J, Krisnapuram R, Pal NR. Pattern Recognition. In: Bezdek JC, Keller J, Krisnapuram R, Pal NR, editors. Fuzzy Models and Algorithms for Pattern Recognition and Image Processing. Boston, MA: Springer US; 1999. p. 1-10.

22.	Dunn JC. A Fuzzy Relative of the ISODATA Process and Its Use in Detecting Compact Well-Separated Clusters. Journal of Cybernetics. 1973;3(3):32-57.

23.	Zhu X, Goldberg AB. Introduction to Semi-Supervised Learning. Synthesis Lectures on Artificial Intelligence and Machine Learning. 2009;3(1):1-130.

24.	K AV. What Is Machine Learning: Definition, Types, Applications And Examples 2019 [

25.	Sutton RS, Barto AG, Williams RJ. Reinforcement learning is direct adaptive optimal control. IEEE Control Systems Magazine. 1992;12(2):19-22.

26.	Puterman ML. Markov decision processes. Handbooks in operations research and management science. 1990;2:331-434.

27.	Rummery GA, Niranjan M. On-line Q-learning using connectionist systems: Citeseer; 1994.

28.	LeCun Y, Bengio Y, Hinton G. Deep learning. Nature. 2015;521(7553):436-44.

29.	Yao L, Ge Z. Deep learning of semisupervised process data with hierarchical extreme learning machine and soft sensor application. IEEE Transactions on Industrial Electronics. 2017;65(2):1490-8.

30.	Rawat AS. Top 8 Machine learning Models Jun 08, 2021 [Available from: https://www.analyticssteps.com/blogs/top-8-machine-learning-models.

31.	Yadev P. Decision Tree in Machine Learning Medium; Nov 13,2018 [Available from: https://towardsdatascience.com/decision-tree-in-machine-learning-e380942a4c96.

32.	Savaram R. What Is Artificial Neural Network and How It Works?

February 15th 2021 [Available from: https://mindmajix.com/artificial-neural-network-and-how-it-works.

33.	Zell A. Simulation neuronaler netze: Addison-Wesley Bonn; 1994.

34.	Schmidhuber J. Deep learning in neural networks: An overview. Neural networks. 2015;61:85-117.

35.	Valueva MV, Nagornov N, Lyakhov PA, Valuev GV, Chervyakov NI. Application of the residue number system to reduce hardware costs of the convolutional neural network implementation. Mathematics and Computers in Simulation. 2020;177:232-43.

36.	Gonzalez Js, Yu W. Non-linear system modeling using LSTM neural networks. IFAC-PapersOnLine. 2018;51(13):485-9.

37.	phi M. Illustrated Guide to LSTM’s and GRU’s: A step by step explanation. Sept 24,2018.

38.	Hochreiter S, Schmidhuber J. Long Short-Term Memory. Neural Computation. 1997;9(8):1735-80.

39.	Graves A, Mohamed A-r, Hinton G, editors. Speech recognition with deep recurrent neural networks. 2013 IEEE international conference on acoustics, speech and signal processing; 2013: Ieee.

40.	Narendra KS, Parthasarathy K. Gradient methods for the optimization of dynamical systems containing neural networks. IEEE Transactions on Neural networks. 1991;2(2):252-62.

41.	Bai J-W, Xiao H-W, Ma H-L, Zhou C-S. Artificial Neural Network Modeling of Drying Kinetics and Color Changes of Ginkgo Biloba Seeds during Microwave Drying Process. Journal of Food Quality. 2018;2018:3278595.

42.	Momenzadeh L, Zomorodian A, Mowla DJJoAS, Technology. Applying artificial neural network for drying time prediction of green pea in a microwave assisted fluidized bed dryer. 2012;14(3):513-22.

43.	Ali Motavali  GHN, Solayman Abbasi ,Saeid Minaei , Abdurrahman Ghaderi. Microwave-vacuum drying of sour cherry: comparison of mathematical models and artificial neural networks. July–August 2013).

44.	Loukas S. Everything you need to know about Min-Max normalization: A Python tutorial: Medium; May 28,2020 [Available from: https://towardsdatascience.com/everything-you-need-to-know-about-min-max-normalization-in-python-b79592732b79.

45.	Khandelwal R. How to solve randomness in an neural artificial neural network Medium; Jan 31,2020 [Available from: https://towardsdatascience.com/how-to-solve-randomness-in-an-artificial-neural-network-3befc4f27d45.

46.	Alhamid M. LSTM and bidirectional LSTM for regression June 26 [Available from: https://towardsdatascience.com/lstm-and-bidirectional-lstm-for-regression-4fddf910c655.

47.	Keras  LSTM tutorial -How to easily build a powerful deep learning language model February 2018 [Available from: https://adventuresinmachinelearning.com/keras-lstm-tutorial/.

48.	Jupyter.

APPENDICES

Appendix A

LSTM Code for optimization of  parameters and implementation using Python on Jupyter

import tensorflow as tf

import pandas as pd

import numpy as np

import random as python_random

import matplotlib.pyplot as plt

from sklearn.preprocessing import MinMaxScaler

import time

from sklearn.model_selection import train_test_split

from sklearn.metrics import r2_score

from tensorflow.keras.optimizers import Adam

# For LSTM model

from tensorflow import keras

#from keras.models import Sequential

#from keras.layers import Dense

#from keras.layers import LSTM

#from keras.layers import Dropout

from tensorflow.keras import Sequential,layers,callbacks

from tensorflow.keras.layers import Dense,LSTM,Dropout,GRU,Bidirectional

from tensorflow.keras.callbacks import EarlyStopping

from tensorflow.keras.models import load_model

from openpyxl import load_workbook

from numpy.random import seed

seed(123)

python_random.seed(123)

tf.random.set_seed(1234)

#df = pd.read_csv('df1.csv')

df = pd.read_excel('FULLDATA.xls')

#X = df.iloc[:, :-1].values

#y = df.iloc[:, -1].values

#y = np.reshape(-1, 1)

X = df

y = df.loc[:,['Tempertaure ']]

X_test = X[4129:4679]

y_test = y[4129:4679]

X_train= X[0:4128]

y_train =y[0:4128]

dfwhole = pd.read_excel('Wholemilktemp2.xls')

Xwhole = dfwhole

ywhole = dfwhole.loc[:,['Tempertaure ']]

dfskim = pd.read_excel('skimmilktemp2.xls')

Xskim = dfskim

yskim = dfskim.loc[:,['Tempertaure ']]

# Scale features

s1 = MinMaxScaler(feature_range =(-1, 1))

#Xs = s1.fit_transform(df[['Tair', 'RH', 'Vel', 'Size', 'Tsec', 'L', 'P', 'F', 'Tdrop']])

Xs = s1.fit_transform(X_train)

#Xs = s1.fit_transform(df(X)) alternative representation

#Scale predicted value

s2 = MinMaxScaler(feature_range=(-1, 1))

#Ys = s2.fit_transform(df[['Tdrop']])

Ys = s2.fit_transform(y_train)

#Ys = s2.fit_transform(df(y)) alternative representation

def layer1(shape1,shape2,j):

model = Sequential()

model.add(LSTM(units=j, return_sequences=True, \

input_shape=(shape1,shape2)))

model.add(Dropout(0.2))

model.add(LSTM(units=j))

model.add(Dropout(0.2))

model.add(Dense(units=1))

model.compile(optimizer = 'adam', loss = 'mean_squared_error',\

metrics = ['accuracy'])

return model

def layer2(shape1,shape2,j):

model = Sequential()

model.add(LSTM(units=j, return_sequences=True, \

input_shape=(shape1,shape2)))

model.add(Dropout(0.2))

model.add(LSTM(units=j, return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j))

model.add(Dropout(0.2))

model.add(Dense(units=1))

model.compile(optimizer = 'adam', loss = 'mean_squared_error',\

metrics = ['accuracy'])

return model

def layer3(shape1,shape2,j):

model = Sequential()

model.add(LSTM(units=j,return_sequences=True, \

input_shape=(shape1,shape2)))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j))

model.add(Dropout(0.2))

model.add(Dense(units=1))

model.compile(optimizer = 'adam', loss = 'mean_squared_error',\

metrics = ['accuracy'])

return model

def layer4(shape1,shape2,j):

model = Sequential()

model.add(LSTM(units=j,return_sequences=True, \

input_shape=(shape1,shape2)))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j))

model.add(Dropout(0.2))

model.add(Dense(units=1))

model.compile(optimizer = 'adam', loss = 'mean_squared_error',\

metrics = ['accuracy'])

return model

def layer5(shape1,shape2,j):

model = Sequential()

model.add(LSTM(units=j,return_sequences=True, \

input_shape=(shape1,shape2)))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j,return_sequences=True))

model.add(Dropout(0.2))

model.add(LSTM(units=j))

model.add(Dropout(0.2))

model.add(Dense(units=1))

model.compile(optimizer = 'adam', loss = 'mean_squared_error',\

metrics = ['accuracy'])

return model

for window in range(22,23) :

#window = 60

X = []

Y = []

for i in range(window,len(Xs)):

X.append(Xs[i-window:i,:])

Y.append(Ys[i])

# Reshape data to format accepted by LSTM

X, Y = np.array(X), np.array(Y)

# Reshaping

#X = np.reshape(X, (X.shape[0], X.shape[1], 9))

for j in range(2,30,2):

for k in range (1,6):

if (k==1):

model=layer1(X.shape[1],X.shape[2],j)

elif(k==2):

model=layer2(X.shape[1],X.shape[2],j)

elif(k==3):

model=layer3(X.shape[1],X.shape[2],j)

elif(k==4):

model=layer4(X.shape[1],X.shape[2],j)

else:

model=layer5(X.shape[1],X.shape[2],j)

# Allow for early exit

es = EarlyStopping(monitor='loss',mode='min',verbose=1,patience=10)

# Fit (and time) LSTM model

t0 = time.time()

#history = model.fit(X, Y, epochs=500, validation_split=0.1, verbose=0)

history = model.fit(X, Y, epochs = 1000, batch_size = 32, callbacks=[es], verbose=1)

t1 = time.time()

print('Runtime: %.2f s' %(t1-t0))

# Plot loss

#plt.figure(figsize=(8,4))

#plt.semilogy(history.history['loss'])

#plt.xlabel('epoch'); plt.ylabel('loss')

#plt.savefig('Tdrop_loss.png')

#model.save('model.h5')

HISTORY = history.history['loss']

# Verify the fit of the model

Yp = model.predict(X)

Yp = Yp.reshape((Yp.shape[0],Yp.shape[1]))

# un-scale outputs

Yu = s2.inverse_transform(Yp)

Ym = s2.inverse_transform(Y)

Xts = s1.transform(X_test)

Yts = s2.transform(y_test)

Xti = []

Yti = []

for i in range(window,len(Xts)):

Xti.append(Xts[i-window:i,:])

Yti.append(Yts[i])

# Reshape data to format accepted by LSTM

Xti, Yti = np.array(Xti), np.array(Yti)

#Xti = np.reshape(Xti, (Xti.shape[0], Xti.shape[1], 9))

# Verify the fit of the model

Ytp = model.predict(Xti)

#Ytp = Ytp.reshape((Ytp.shape[0],Ytp.shape[1]))

# un-scale outputs

Ytu = s2.inverse_transform(Ytp)

Ytm = s2.inverse_transform(Yti)

#Loss

#plt.figure(figsize=(10,6))

#plt.subplot(2,1,1)

#plt.plot(Ytu,'r-',label='LSTM Predicted')

#plt.plot(Ytm,'k--',label='Measured')

#plt.legend()

#############################################

# Using predicted values to predict next step

Xtsq = Xts.copy()

for i in range(window,len(Xtsq)):

Xin = Xtsq[i-window:i].reshape((1, window, 10))

Xtsq[i][0] = model.predict(Xin)

Yti[i-window] = Xtsq[i][0]

Ytun = (Yti - s2.min_[0])/s2.scale_[0]

Ytun = s2.inverse_transform(Yti)

R2SCORETESTPRE = r2_score(Ytm, Ytun)

################################################

#whole milk

Xts1 = s1.transform(Xwhole)

Yts1 = s2.transform(ywhole)

Xti1 = []

Yti1 = []

for i in range(window,len(Xts1)):

Xti1.append(Xts1[i-window:i,:])

Yti1.append(Yts1[i])

# Reshape data to format accepted by LSTM

Xti1, Yti1 = np.array(Xti1), np.array(Yti1)

#Xti = np.reshape(Xti, (Xti.shape[0], Xti.shape[1], 9))

# Verify the fit of the model

Ytp1 = model.predict(Xti1)

#Ytp = Ytp.reshape((Ytp.shape[0],Ytp.shape[1]))

# un-scale outputs

#wholemilk predicted

Wmp = s2.inverse_transform(Ytp1)

#wholemilk measured

Wmm = s2.inverse_transform(Yti1)

########################################################

#skim milk

Xts2 = s1.transform(Xskim)

Yts2 = s2.transform(yskim)

Xti2 = []

Yti2 = []

for i in range(window,len(Xts2)):

Xti2.append(Xts2[i-window:i,:])

Yti2.append(Yts2[i])

Xti2, Yti2 = np.array(Xti2), np.array(Yti2)

# Verify the fit of the model

Ytp2 = model.predict(Xti2)

#Ytp = Ytp.reshape((Ytp.shape[0],Ytp.shape[1]))

# un-scale outputs

#skimemilk predicted

Skp = s2.inverse_transform(Ytp2)

#skimmilk measured

Skm = s2.inverse_transform(Yti2)

print('The R2 score of the Training set is :\t{:0.3f}'.format(r2_score(Ym, Yu)))

print('The R2 score of the Testing set is :\t{:0.3f}'.format(r2_score(Ytm, Ytu)))

R2SCORETRAIN = r2_score(Ym,Yu)

R2SCORETEST = r2_score(Ytm, Ytu)

R2SCOREWHOLEMILK = r2_score(Wmp,Wmm)

R2SCORESKIMMILK = r2_score(Skp,Skm)

print(k,window,j,R2SCORETRAIN)

print(k,window,j,R2SCORETEST)

print(k,window,j,R2SCOREWHOLEMILK)

print(k,window,j,R2SCORESKIMMILK)

print(k,window,j,R2SCORETESTPRE)

name ="window="+str(window)+","+"Neurons="+str(j)+","+"layers="+str(k)

ytu = pd.DataFrame(Ytu,columns=['LactosePredicted'])

ytm = pd.DataFrame(Ytm,columns=['LactoseMeasured'])

ytun = pd.DataFrame(Ytun,columns=['LactosePretoPre'])

wmm = pd.DataFrame(Wmm,columns=['WholemilkMeasured'])

wmp = pd.DataFrame(Wmp,columns=['WholemilkPredicted'])

skm = pd.DataFrame(Skm,columns=['SkimmilkMeasured'])

skp = pd.DataFrame(Skp,columns=['SkimmilkPredicted'])

HISTORY1 =pd.DataFrame(HISTORY,columns=['LOSS'])

# R2_TRAIN = pd.DataFrame(R2SCORETRAIN,columns=['R2SCOETRAIN'])

# R2_TEST = pd.DataFrame(R2SCORETEST,columns=['R2SCOETEST'])

# R2_TESTPre = pd.DataFrame(R2SCORETESTPRE,columns=[' R2SCORETESTPRE'])

# PredictedDF = pd.concat([ytm,ytu,ytun,HISTORY1,R2_TRAIN,R2_TEST,R2_TESTPre], axis=1)

PredictedDF = pd.concat([ytm,ytu,ytun,wmm,wmp,skm,skp,HISTORY1],axis=1)

#PredictedDF.to_excel(r'C:\Users\AKINYEMI\Desktop\500l\Orignal project\SENT\DATA.xlsx', sheet_name = name , index = False)

#PredictedDF.to_excel(r'C:\Users\AKINYEMI\Desktop\500l\Orignal project\SENT\name.xlsx', sheet_name = name , index = False)

file_name = name+".xlsx"

PredictedDF.to_excel(file_name)

# new dataframe with same columns

df = pd.DataFrame({'WINDOW': [window],

'LAYERS': [k],

'NEURON': [j],

'R2TRAIN': [R2SCORETRAIN],

'R2TEST': [R2SCORETEST],

'R2PRETOPRE': [R2SCORETESTPRE],

'R2SCOREWHOLEMILK':[R2SCOREWHOLEMILK],

'R2SCORESKIMMILK':[R2SCORESKIMMILK]})

writer = pd.ExcelWriter('demoB.xlsx', engine='openpyxl')

# try to open an existing workbook

writer.book = load_workbook('demoB.xlsx')

# copy existing sheets

writer.sheets = dict((ws.title, ws) for ws in writer.book.worksheets)

# read existing file

reader = pd.read_excel(r'demoB.xlsx')

# write out the new sheet

df.to_excel(writer,index=False,header=False,startrow=len(reader)+1)

writer.close()